# SAMRT PARKING SYSYTEM AND FIRE DETECTION

TEAM MEMBERS:

- JEEVALAKSHMIV(23ECE071)

- SUVEDHA B(23ECE087)

- SWETHA S(23ECE089)

INTRODUCTION

With the growth of urban areas, parking management and safety in public spaces have become major challenges. This project provides a smart IoT-based solution that helps in monitoring parking slot availability while also ensuring safety by detecting fire, gas leaks, and high temperatures.

OBJECTIVE

To design and implement an IoT-based system that not only monitors parking slot availability but also ensures safety by detecting fire, gas leakage, and abnormal temperature rise. The system provides real-time updates and sends notifications to the user through the Blynk IoT platform.

COMPONENTS USED

- ESP32 Microcontroller

- Ultrasonic Sensors (HC-SR04)

- DHT11 Temperature Sensor

- MQ2 Gas Sensor

- 16x2 LCD with I2C Module

- Buzzer and LED

- Jumper Wires and Breadboard

- Blynk IoT App / Web Dashboard

CIRCUIT DAIGRAM

WORKING

- Parking Monitoring

    - Ultrasonic sensors are placed in parking slots.

    - If the measured distance is less than a set threshold, the slot is    marked as Occupied; otherwise, it is Available.

- Temperature Monitoring

  - The DHT22 continuously checks the surrounding temperature.

  - If the temperature exceeds a safe level, an overheat alert is generated.

- Fire and Gas Detection

  - The flame sensor triggers an alert if fire is detected.

  - The MQ2 sensor monitors smoke and gas levels; when they cross the threshold, a gas alert is triggered.

- Alerts and Outputs

  - LCD shows slot status and alert messages in real-time.

  - Buzzer and LED activate to provide instant warning signals.

  - ESP32 sends the data to the Blynk IoT cloud, where users can monitor on:

    - Mobile dashboard

    - Web dashboard

  - If a dangerous situation is detected, the user receives a push notification on their phone.

FEATURES

- Real-time parking slot monitoring

- Multi-sensor safety integration (fire, gas, temperature)

- Alerts via LCD, buzzer, LED, and Blynk app

- Remote monitoring on both web and mobile

- IoT-based scalable solution for smart cities

TOOLS USED

- Arduino IDE – Programming & uploading

- Wokwi – Circuit simulation

- Blynk IoT – Cloud platform for monitoring